ASIC A0 EPTG specification

23 March 2017

Revision 0.1.0

Intel Top Secret

Table 1: Revision history

| Matlab Version | Revision Number | Revised by | Description | Revision Date |
| --- | --- | --- | --- | --- |
| 2016B | 0.1.0 | Yoni Chechik | Initial version | March 23, 2017 |

Contents

1	Introduction	3

2	Interfaces	4

3	Detailed description	5

4	Registers	6

5	Memory	8

6	Performance	9

7	Test plan	10

7.1	General randomization	10

8	Constraints	12

Tables

Table 1: Revision history	1

Table 2: Registers	7

Introduction

EPTG stands for ‘external pattern generator’ and it’s a class of registers (not an actual block) designed to simulate different inputs to the ASIC pipe.

The registers configure different parts of the input in the Pipe.patternGenerator function and builds *.ivs + config.csv + calib.csv files that can later be used in the pipe itself.

Interfaces

NA

Detailed description

NA

Registers

| Name | Size | Default | Range | Special values/ description |
| --- | --- | --- | --- | --- |
| EPTGseed | 1 | 1 | 0:2^32-1 | Seed to control randomness (e.g. added noise) so results could be duplicate |
| EPTGzImageType | 1 | 2 | 1:5 | Generate z image to scan: - Plane in height EPTGmaxZ - 16 cubes with face perpendicular to plane 3:5- faces not perpendicular to plane |
| EPTGmaxZ | 1 | 3000 | 0.0:8000.0 | Max distance of z image |
| EPTGminZ | 1 | 500 | 1.0:4000.0 | min distance of z image |
| EPTGnoiseLevel | 1 | 0.4 | 0.0:10.0 | Amount of system noise. assuming max rx signal = 1 (distance 0) |
| EPTGframeRate | 1 | 60 | [2.0:66.0]; [100.0:140.0]; 1000.0 | pattern generator output frame rate (1000 value for debug only) |
| EPTGmirrorFastFreq | 1 | 2000 | 18000.0:22000.0 | mirror fast frequency |
| EPTGslowscanType | 1 | 2 | 0:2 | Type of scan in slow axis: - Linear - Sine - atan |
| EPTGirImageType | 1 | 3 | 1:3 | Generate IR image to scan: - Wall - Checkerboard - Random cubes |
| EPTGmultiFocalROI_00[0:4] | 1 | 0.0 | -2048.0:2047.0 | --- |
| EPTGnMaxSamples | 1 | 1600000000 | 1000000.0:16000000000.0 | Max number of scanned data points (number of fast channel samples). The actual number of samples will be the minimum between this register and EPTG.frameRate*1e9*chunkFreq When chunkFreq is double(regs.GNRL.sampleRate)/(64*double(regs.MTLB.txSymbolLength)) |
| EPTGslowCouplingFactor | 1 | 0 | 0.0:0.002 | Coupling percentage to fast axis: x axis will be change as a function of the fast frequency for values larger than 0. |
| EPTGsampleJitter | 1 | 0.1 | 0.0:1.0 | fast path sampling jitter rms(nsec) |
| EPTGinputAsRange | 1 | 0 | 0:1 | set input as range |
| EPTGldonFallIncidents | 1 | 0 | 0:4 | number of times ld_on falls during frame |
| EPTGldonmaxFallTime | 1 | 5000 | 0:100000 | max fall time in nsec |
| EPTGirSampleFreq | 1 | 125 | 62.5;125.0 | ir(slow channel) sample frequency [Mhz] |

Table 2: Registers

Memory

NA

Performance

NA

Test plan

## General randomization

Table 3 describes the general test randomization distribution.

| Name | Value | Distribution |
| --- | --- | --- |

Table 6: General randomization distribution

TBD

Constraints

- [EPTGminZ] <=[EPTGmaxZ]

- [EPTGminZ] >=0

- [EPTGnMaxSamples] == floor([EPTGnMaxSamples])